[Test Procedure: Network Storm DoS Test]

Avikus

Accept by: ___________________________

Tested by: Yang Jeongwon

Avikus

Date: 2026-05-22

[ Revision History ]

| Documents Number. | Revision Number. | Revision Date | Description |
| --- | --- | --- | --- |

[ Operator Information ]

| Operator | Position | Name | Signature |
| --- | --- | --- | --- |

[ Caution ]

AVIKUS PROPRIETARY DATA This document contains confidential proprietary information. Restricted to use only by Avikus employees or other persons authorized by Avikus in writing to use it. Since it contains Avikus trade secrets, you cannot view, use, copy, distribute, or disclose this material without the prior written approval of Avikus.

Contents

1.	Pre-Test Checklist	4

2.	Test Contents & Procedure	4

2.1	System Health Check Test	4

2.2	Network Storm (DoS) Test	5

Pre-Test Checklist

| No | Check Item | Confirmation |
| --- | --- | --- |
| 1 | HiNAS system powered ON and operating normally | ☑ Pass ☐ Fail |
| 2 | Fortigate firewall powered ON and GUI access available. | ☑ Pass ☐ Fail |
| 3 | Firewall monitoring environment prepared (bandwidth widget, log view) | ☑ Pass ☐ Fail |
| 4 | DoS / Network Storm related firewall policies configured and enabled. | ☑ Pass ☐ Fail |
| 5 | Logging enabled for traffic events | ☑ Pass ☐ Fail |
| 6 | Test laptop prepared (DoS script, IP configuration). | ☑ Pass ☐ Fail |
| 7 | Network connection and target IP (HiNAS/BMS) verified | ☑ Pass ☐ Fail |
| 8 | Test monitoring screens ready (HiNAS UI, firewall dashboard). | ☑ Pass ☐ Fail |

Test Contents & Procedure

System Health Check Test

| Relevant Standard | Standard No. |  |  |
| --- | --- | --- | --- |
|  | Article. |  |  |
| Purpose | The purpose of this test is to verify the system and hardware installation status before executing the Network Storm (DoS) test. |  |  |
| Procedure of test |  | Pass | Fail |  |
| - Verify system installation status. - HiNAS system is installed on board - Firewall is installed and physically connected to HiNAS - L2 switch for HiNAS / BMS network is installed - Verify system health status. - HiNAS system boots normally without error - No abnormal alarm or fault message is displayed on HiNAS UI |  | Pass | Pass |
| Comment / Recommended |  |  |  |

Network Storm (DoS) Test

| Relevant Standard | IACS UR E27. |  |  |
| --- | --- | --- | --- |
|  | Cyber Resilience of On-board Systems and Equipment |  |  |
| Purpose | The purpose of this test is to verify that the HiNAS system maintains normal operation when excessive network traffic (Network Storm / DoS condition) is generated, and that such abnormal traffic is properly detected and logged by the firewall. |  |  |
| Procedure of test |  | Pass | Fail |  |
| - Confirm that the HiNAS system is operating normally before the test. - Connect the test laptop to the L2 switch connected to the BMS network, and configure the test laptop IP address as the BMS IP. - Set the target IP address to the HiNAS system. - Execute the DoS (Network Storm) traffic generation script with the following configuration: - Protocol: UDP - Target IP: 10.0.100.20 (HiNAS) - Source IP: 10.0.50.12 (BMS) - Target Port: 5005 - Packet Size: 1024 bytes - Traffic Load: configured to generate approximately 300 Mbps of traffic - During the traffic generation, verify the following: - Firewall bandwidth usage increases and DoS-related detection is observed - HiNAS system continues normal operation without malfunction - No abnormal delay, freeze, or loss of display is observed on the HiNAS UI - Stop the DoS traffic after sufficient observation time. - Review firewall logs and HiNAS system logs to confirm that Network Storm / DoS events are properly recorded. |  | Pass | Pass |
| Comment / Recommended |  |  |  |