Factory Acceptance Test Procedure

| PRODUCT NAME | HiNAS Control Standard | SERIAL NUMBER | RMS26-0005 |
| --- | --- | --- | --- |
| YARD | HMM | HULL NO. | IVY |
| CLASS | DNV | DATE | 2026-05-04 |
| INSPECTOR | Chong, Sang Ok |  |  |

AVIKUS PROPRIETATY DATA

This document cotains confidential proprietary information. 
Restricted to use only by Avikus employees or other persons authorized by Avikus in writing to use it.
Since it cotains Avikus trade secrets, you cannot view, use, copy, distribute, or disclose this material
without the prior written approval of Avikus.

| Version | Description | Author/EDITED | Revised/Approved | Date |
| --- | --- | --- | --- | --- |
| 1.0 | First Draft | Dongheun Kang, Jinhwan Park | Sangdeok Choi | 2025-07-02 |
| 1.1 | Minor Update | Dongheun Kang, Jinhwan Park | Sangdeok Choi | 2025-07-03 |
| 1.2 | Comment Update | Dongheun Kang | Sangdeok Choi | 2025-07-10 |

Note: In this document, ‘HiNAS Control’ is used to denote both HiNAS Control and HiNAS Control Standard.

- Overview

  - Purpose

This document provides a comprehensive description of the Factory Acceptance Test (FAT) for HiNAS Control Standard. The following items are verified through this document:

- Verify that the configuration and quantity of the HiNAS Control Standard hardware are the same as those shown in the drawings (Appendix A) submitted with the MED-B certificate.

- Verify that the Serial Numbers of the hardware listed in the DoC document match those of the hardware actually installed onboard.

- Verify that the software version recorded in the SW install package used for installation matches the version in the actual product.

- Verify that the data from all navigation equipment connected to the overall system are received correctly and that the software functions operate properly.

- Verify that the connected hardware performs its functions correctly.

  - Test Environment

Unless otherwise specified, the following test conditions shall be applied uniformly during the Factory Acceptance Test (FAT):

∙ Temperature: 25 °C ± 10 °C
∙ Atmospheric Pressure: Normal ambient pressure
∙ Humidity: 20% to 80%

  - Test Site

The test shall be conducted on board

  - Applicable Document

    - EC-TYPE EXAMINATION CERTIFICATE (MODULE B)

MEDB0000BWN (Revision no. 1)

    - Drawing

Approval Drawing of the vessel subject to FAT. (Appendix A.)

    - Inspection Report

Inspection Report for the item subject to inspection

    - SW Installation Report

SW Installation Report for the item subject to software installation

- Information about the test setup

  - Components List (MEDB0000BWN)

The components list specified in the EC-Type Examination Certificate is as follows.

| No. | Type | Type Designation | Maker | Note/description/limitation |
| --- | --- | --- | --- | --- |
| 1 | Server | Optiplex XE4 Tower with damping kit TVK-XE4-TV or TVK-XE4-TH | Tetradyne | At least two per installation. |
| 2 | Client | Optiplex XE4 Tower with damping kit TVK-XE4-TV or TVK-XE4-TH | Tetradyne | Optional. In server/client config, two per installation |
| 3 | Monitor | TIMX-W270 or TIMX-W320 or TIMX-UW584 | Tetradyne | System may be installed with two monitors in same installation. |
| 4 | Keyboard with Trackball | RKTE85B2448- W-MC1 | NSI | To be installed with each delivery |
| 5 | Serial to Ethernet | Nport-5450i | Moxa | Minimum one per installation |
| 6 | Network switch | EDS-G508E | Moxa | Minimum one per installation |
| 7 | Firewall | FGR-60F | Fortinet | One per installation |
| 8 | Selector switch | MPH 731 | Yokogawa | One per installation |
| 9 | Dimmer | MKD002 | Yokogawa | One per installation |
| 10 | UPS | 9SX1000IM or 9SX3000IM or 9PX1500iRTM | Eaton | Minimum one UPS per installation. |
| 11 | Camera unit, incl. EO cameras and IR cameras | HiNAS Navigation | Avikus | Optional |
| 12 | Junction box incl. Single Board Computer |  |  |  |

  - Components’ Software Version

| No. | Type | Type Designation | Software Version |
| --- | --- | --- | --- |
| 1 | Heading Control System | PR-9000 | HCCPU VER.2.02 |
| 2 | Remote Control System | M-800-VII | NAS NX2 VER 2.0.1 |
| 3 | ECDIS | PM3D2 | PM4D |
| 4 | HiNAS Control Standard |  | V2.0.4 |
| 5 | Operating System | ubuntu | 22.04 |

  - Test Preparation

    - Verification

Verify whether there are any non-compliant components by comparing the drawings of the FAT target vessel with the EC-Type Examination Certificate.

    - Installation

All components shall be fully installed on the test bed prior to testing.

    - Wiring

The test items shall be wired according to the wiring diagram of the FAT target vessel.
The selector switch and dimmer box are excluded from the wiring, as there are no external interface devices.

    - Power Input

After verifying that there are no wiring errors, system power shall be applied.

    - Software Installation

The SW is installed by ‘Manual: HiNAS Control Standard Installation Manual’ on MED-B cert.

- Factory Acceptance Test

  - Visual Inspection Test

    - Visual Inspection

| No. | Test | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pass | Fail |
| 1 | Visually inspect the prepared test items for any visible damage or deformation | No damage or deformation |  | v |  |

    - Verification of conformity

Verify that the FAT items conform to the Type Designation specified in the EC-Type Examination Certificate

※ For items not applicable, indicate "N/A" in the Result column and skip the test.

| No. | Type | Type Designation | Qty | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pass | Fail |
| 1 | Server | Optiplex XE4 Tower with damping kit TVK-XE4-TV or TVK-XE4-TH | 1 | Type Designation conformity | Type Designation : Optiplex XE4 Tower with damping kit TVK-XE4-TH SER. NO. : 1L6F4G4 | v |  |
| 2 | Client | Optiplex XE4 Tower with damping kit TVK-XE4-TV or TVK-XE4-TH | 1 | Type Designation conformity | Type Designation : Optiplex XE4 Tower with damping kit TVK-XE4-TV SER. NO. : CL6F4G4 | v |  |
| 3 | Monitor | TIMX-W270 | 1 | Type Designation conformity | Type Designation : TIMX-W270 SER. NO. : TM260313-069 | v |  |
| 4 | Keyboard with Trackball | RKTE85B2448- W-MC1 | 1 | Type Designation conformity | Type Designation : RKTE85B2448-W-MC1 SER. NO. : 2026.42300.020 | v |  |

| No. | Type | Type Designation | Qty | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pass | Fail |
| 5 | Serial to Ethernet | Nport-5450i | 1 | Type Designation conformity | Type Designation : Nport5450i SER. NO. : TBFBE1072335 | v |  |
| 6 | Network switch | EDS-G508E | 2 | Type Designation conformity | Type Designation : EDS-G508E SER. NO. : TBFCD1009081 TBFCD1051373 | v |  |
| 7 | Firewall | FGR-60F | 1 | Type Designation conformity | Type Designation : FGR-60F SER. NO. : LNSBFJDJBI | v |  |
| 8 | Selector switch | MPH 731 | 1 | Type Designation conformity | Type Designation : MPH 731 SER. NO. : 1067 | v |  |
| 9 | Selector Switch Dimmer | MKD002 | 1 | Type Designation conformity | Type Designation : MKD002 SER. NO. : 11892 | v |  |
| 10 | UPS | 9SX1000iM | 1 | Type Designation conformity | Type Designation : 9SX1000RTM SER. NO. : GC02V05185 | v |  |

| No. | Type | Type Designation | Qty | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pass | Fail |
| 11 | Camera unit, incl. EO cameras and IR cameras | HiNAS Navigation | 1 | Type Designation conformity | Type Designation: HiNAS Navigation SER. NO. : Camera unit : SC0004-NCH01-26021 | v |  |
| 12 | Junction box, incl. Single Board Computer |  | 1 | Type Designation conformity | Type Designation: HiNAS Navigation SER. NO. Junction box : SC0005-NJB01-26041 Single board computer : NAS-HUCX08250031 EMC -filter : SZ0014-EMC01-140 SMPS : 2025-157(R) | v |  |

Verify that the types and quantities of equipment installed on board the vessel match those specified in the Declaration of Conformity and the submitted drawings.

| No. | Test | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pass | Fail |
| 1 | Verify that the configuration and quantity of the HiNAS Control Standard hardware are the same as those shown in the drawings |  |  | v |  |
| 2 | Verify that the Serial Numbers of the hardware listed in the DoC document match those of the hardware actually installed onboard. |  |  | v |  |

  - Functional Test

    - Dimmer

Verify the operation of the monitor's dimming function

- For monitors not applicable, indicate "N/A" in the Result field and omit the test

| No. | Test |  | Criteria | Result |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pass | Fail |
| 1 | Verify whether the brightness can be adjusted by pressing the “-” and “+” buttons on the OSD Remote Control Unit of model TIMX-UW584. |  | Brightness adjustable |  | v |  |
|  | For reference |  |  |  |  |  |

| No. | Test |  | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pass | Fail |
| 2 | Verify whether the brightness can be adjusted by pressing the “-” and “+” buttons located at the lower right corner of models TIMX-W270 and TIMX-W320 |  | Brightness adjustable |  | v |  |
|  | For reference |  |  |  |  |  |

    - Keyboard with Trackball

| No. | Test | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pass | Fail |
| 1 | Verify that the keyboard inputs function correctly. |  |  | v |  |
| 2 | Verify that the mouse clicks function correctly. |  |  | v |  |

    - Software Installation Check Test

| No. | Test |  | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pass | Fail |
| 1 | Verify that HiNAS Control Standard SW is running on the monitor. |  | HiNAS Control Standard SW is running on the monitor. |  | v |  |
| 2 | Double-click ‘Avikus HiNAS’ Icon on the desktop to open the login screen, log in with the provided default account |  | Login operates normally. |  | v |  |
| 3 | Reboot the HiNAS Control Standard client PC and verify that the web browser automatically launches upon boot. |  | HiNAS Control Standard automatically launches after reboot. |  | v |  |
| 4 | Verify the Ubuntu OS version of the server and client PCs using the ‘lsb_release -a’ command in the terminal. |  | Ubuntu 22.xx (x is 04 or higher) |  | v |  |
|  | For reference |  |  |  |  |  |
| 5 | Verify that the Version text at the lower left corner of HiNAS Control Standard matches the SW installation package. |  | Version v1.x.x (x is 0 or higher) |  | v |  |
|  | For reference |  |  |  |  |  |

    - System Integration Test

All signals are applied to the firewall, network switch, and Serial to Ethernet device to verify the proper operation of the HiNAS Control Standard whole system. Software functions are being checked, and the related items can be verified in the ‘Onboard Test Procedure document.’

- Performance Tests

- System Health Check Test

- Basic Functionality Test

- Route Planning Test

- Autopilot fail test

- Sensor interface fail test

- BMS Fail Test

- Power Fail Test

- Operation Mode Change Test – Mode Selection Unit

- Operation Mode Change Test – SW Button

    - Power failure test

Verify the operation of the UPS backup power when the input power is disconnected

| No. | Test | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pass | Fail |
| 1 | Upon disconnection of the input power to the UPS, the system shall remain operational via the UPS backup power. | Normal Operation upon UPS Input Power Disconnection |  | v |  |

    - Verification of Serial Number

| No. | Test | Criteria | Result | Judgment |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pass | Fail |
| 1 | Verify the HiNAS Control Standard serial number and the presence of the Wheel Mark by inspecting the nameplate on the server. | Serial Number and Wheel Mark Labeling |  | v |  |

| RESULT |  | A |  |
| --- | --- | --- | --- |
| “A”: Accepts, “B”: Accept with following comments, “C”: Canceled, “R”: Reinspection |  |  |  |
| Signature | Avikus |  |  |

Appendix A.

- Wiring Diagram